The University of Nottingham

SCHOOL OF COMPUTER SCIENCE

A LEVEL 4 MODULE, AUTUMN SEMESTER 2018-2019

ADVANCED COMPUTER NETWORKS

(Module COMP4032 / G54ACN)

Time allowed TWO hours

Answer THREE questions

Candidates may complete the front cover of their answer book and sign their desk card but must NOT write anything else until the start of the examination period is announced.

Answer 3 out of 5 questions

Marks available for sections of questions are shown in brackets in the right-hand margin

Only silent, self-contained calculators with a single-line display are permitted in this examination.

Dictionaries are not allowed with one exception.  Those whose first language is not English may use a dictionary to translate between that language and English provided that neither language is the subject of this examination.

No electronic devices capable of storing and retrieving text, including electronic dictionaries, may be used.

DO NOT turn your examination paper over until instructed to do so

ANSWERS

1. Reliability										 (33 marks)

a) One could argue that a loss estimate of zero indicates that there is no congestion and thus the sending rate should be increased with the maximum possible increase factor until a loss event occurs. Is this true? Justify your answer.

(6)

Marking:

2 points for correctly answering this is not true.

4 points for a detailed justification and an example of how this could be handled. (2 points for correct justification, 2 points for providing an example of how it could be handled)

Answer:

No.

This approach causes instabilities in the sending rate and is very susceptible to a noisy packet drop rate.

TCP Tahoe addressed this by using several techniques:

- RTT variance estimation

- Slow start

- Congestion avoidance and recovery

b) Discuss why you may not want TCP’s features when transporting real-time multimedia data.

(7)

Marking:

3 points for correctly identifying results of mimicking TCP behaviour.

4 points for correctly identifying multimedia streaming applications requirements.

Answer:

Mimicking TCP behaviour (in particular exponential back-off strategy) for multimedia congestion control results in fairness towards TCP but also in very significant oscillations in bandwidth.

However, multimedia streaming applications need to have much lower variation at throughput over time compared to TCP to result in relatively smooth sending rates that are of importance to the end-user perceived quality.

c) Describe three issues that must be addressed when dealing with any network adaptation mechanism.

(9)

Marking:

9 points for correctly listing and explaining the 3 issues (1 point for listing each issue, 2 points for explaining it)

Answer:

- Decisions function: High level overview of decision functions can be:

  - On congestion (overload/packet loss/packet loss increase), decrease the rate immediately, or periodically

  - On absence of congestion (underload/no packet loss, packet loss decrease), increase the rate immediately

- Increase/decrease algorithm:

  - Options for increase phase: (during underload):

    - Constant additive increase rate,

    - Dynamic additive increase rate or

    - Straight jump to the expected value or value calculated by the formula

    - Multiplicative increase rate

  - During congestion, decrease factor can be:

    - Constant multiplicative decrease factor, TCP-like or TCP-similar like.

    - Linear decrease

    - straight jump to the expected value (calculated by the formula)

- Decision frequency

  - Decision frequency specifies how often to change the rate. Based on system control theory, optimal adjustment frequency depends on the feedback delay.

  - The feedback delay is the time between changing the rate and detecting the network’s reaction to that change.

  - It is suggested that equation-based schemes adjust their rates not more than once per RTT.

  - Changing the rate too often results in oscillation whereas infrequent change of the rate leads to an unresponsive behaviour.

d) TCP enables end to end congestion control in the Internet. Real-time multimedia content is often transferred over UDP. Would application level congestion control use RTT or smoothed RTT when estimating packet loss rates? Discuss and justify your answer.

(8)

Marking:

3 points for correctly identifying SRTT as the correct mechanism.

2 points for correctly explaining why SRTT is more appropriate.

3 points for correctly listing the RTT/SRRT algorithms.

Answer:

SRTT is a more appropriate choice for estimated packet loss rates.

In order to adapt to varying and unpredictable network conditions, the timeout is not fixed, but computed based on the algorithm for TCP timeout computation

- Before the first packet is acknowledged and RTT measurement is made, the sender sets the TIMEOUT to a certain initial value.

  - This value is usually 2.5 – 3 seconds for TCP.

  - For real time interactive multimedia traffic, the timeout value should be set to the tolerable delay for interactive conferencing of approximately 0.5 seconds (as recommended by [Brady, P., 1971]).

- When the first RTT measurement is taken the sender sets the smoothed RTT (SRTT), RTT variance (RTTVAR) and TIMEOUT in the following way:

  - SRTT = RTT

  - RTTVAR = RTT/2

  - TIMEOUT = Mu*SRTT + 4*RTTVAR,

    - Where Mu is a constant, which in this implementation is 1.08 (determined after extensive experimentation).

- When subsequent RTT measurements are made the sender sets the RTTVAR, SRTT, TIMEOUT in the following way:

  - RTTVAR = (1 – ¼) * RTTVAR + ¼ * |SRTT – RTT|

  - SRTT = (1 – 1/8)* SRTT + 1/8 * RTT

  - TIMEOUT =Mu*SRTT + 4*RTTVAR

e) What does “packet loss event” mean?

(3)

Marking:

1 point for correctly answering what a “loss event” is.

2 points for explaining how it is calculated.

Answer:

A “loss event” is defined as a number of packets lost within a single RTT.

Real time interactive multimedia approaches typically use the Weighted Loss Interval Average (WLIA).

It relies on using loss events and loss intervals for correct computation of packet loss rate and is in accordance with how TCP performs packet loss calculation.

The number of packets between two consecutive loss events defines a loss interval.

2. VANETs and DTNs (33 marks)

a) What are the two mobility aspects of VANETs that differentiate them from MANETs? Justify your answer.

(5)

Marking:

2 points for correctly listing the two mobility aspects (1 point per aspect)

3 points for explaining the differentiating aspects and correct justification.

Answer:

Predictive mobility: the nodes in a VANET cannot follow arbitrary direction, they have to stay in the road way and cannot suddenly change their direction. These features allow for the design of highly predictive routing protocols. MANETs, in contrast, assume less predictive patterns of movement and cannot be easily scheduled.

High speed mobility: the network mobility in a VANET changes rapidly due to vehicular speeds. This feature can lead to very short connectivity times between vehicles during which low V2V bandwidth can be achieved which leads to vehicles isolation.  MANETs, in contrast, assume low mobility, longer connection times, lower isolation times and more opportunity of data dissemination between the MANET nodes.

Both MANETs and VANETs can result in partitions but VANETs can more often follow the Accordion effect where the topology range rapidly between highly congested and highly disconnected.

b) Compare Beacon and Event safety messages in VANETs. Which of these two types of safety messages will be used to report on an accident?

(3)

Marking:

1 point for correctly identifying the Event safety message as the correct one to be used to report on an accident.

2 points for describing and comparing the Event and Beacon messages usage and features.

Answer:

- The Beacon safety messages

  - The beacon safety messages are periodically issued by vehicles to announce their state such as speed, direction etc. to other neighbouring nodes within communication range.

  - The beacon message allows neighbouring nodes to be aware of their environment and protects them against potential dangers.

- The Event safety messages

  - The event driven messages are sent when an imminent danger has been detected.

  - When detected, the message is disseminated within the areas of high priority. The event driven message should be delivered to nodes with high reliability and less time. This is because a delayed message or a lost message could lead to a loss of life.

The Event safety message will be used to report on an accident.

c) Provide three key assumptions that DTN networks drop and that traditional internet protocols make.

(3)

Marking:

3 points for listing any three of the assumptions listed below (1 point per assumption).

Answer:

- Availability of end to end paths,

- Low RTTs,

- High availability to naming,

- Availability to security services, caching and searching infrastructures to provide locator-based access such as DNS.

d) How and why is the principle of “forwarding” fundamentally changed in DTNs compared to the traditional Internet?

(4)

Marking:

4 points for listing the four changes to “forwarding” principles. (1 point per item).

Answer:

When the connectivity assumptions do not hold:

- Applications break / communication disabled

- Need for fundamentally new communication and security paradigms

- Support interoperability among radically heterogeneous networks and achieve good performance with very large delays and unpredictable loss of connectivity

- DTNs use “store, carry and forward” to transfer data and security messages

e) Is PKI applicable to DTNs? Please justify your answer.

(5)

Marking:

1 points for correctly identifying PKI approaches are not suitable for DTNs.

4 points for justifying the answer and listing two reasons. (2 points each for listing and explaining two reasons).

Answer:

Traditional symmetric cryptography and PKI-based approaches are not suitable for DTNs.

There are two major reasons for this:

- In a PKI, a user authenticates another user’s public key using a certificate signed by a certificate authority (CA)

  - In a disconnected network, without online access to an arbitrary receiver’s public key or certificate, sending an encrypted message on the fly is not possible

- PKIs implement key revocation based on frequently updated online certificate revocation lists (CRLs) posted by CAs.

  - In the absence of instant online access to CRLs, a receiver cannot authenticate a sender’s certificate

High round-trip times and disconnections do not allow frequent distribution of a large number of certificates and encryption keys end-to-end. It would be more scalable to use user’s keys and credentials at neighbouring or nearby nodes.

f) What kind of networks can support interplanetary communications? Identify three characteristics of these networks.

(3)

Marking:

3 points for correctly listing any three of the below characteristics (1 point per item).

Answer:

Interplanetary communication can be supported by DTNs. Main Characteristics of interplanetary networks are:

- High Intermittent Connectivity

- Extremely Long Propagation - Delay: finite speed of light

- Low Transmission Reliability: positioning inaccuracy, limited visibility

- Low Asymmetric Data Rate

- DTNs are designed specifically to support these features.

g) Briefly discuss four mandatory DTN bundle security mechanisms.

(5)

Marking:

4 points for correctly listing and briefly discussing the four mandatory DTN bundle security mechanisms.

1 point for DoS protection mention if a thorough understanding is demonstrated based on class examples and discussions and material from the coursework assignment.

Answer:

Based on the “bundle” protocol:

- Hop-by-hop bundle header integrity

- Hop-by-hop bundle sender authentication

- Access Control (only legitimate applications/users with appropriate permissions may inject bundles)

- Mandatory protection of the DTN infrastructure from unauthorized use - detect illegitimate traffic ASAP and drop it immediately.

  - Limited protection against DoS by detecting illegitimate traffic at its first hop and discarding it immediately.

h) Centrality is a measure of node importance in a graph. Provide two types of graph centralities that may be useful in social DTN networks and explain why this is the case.

(5)

Marking:

2 points for listing correctly two of the five centrality metrics below.

3 points for detailed explanation and discussion.

Answer:

Centrality is a measure of the structural importance of a node in a network.

The most frequently use of centrality metrics for social DTNs include Similarity, Betweeness and Tie Strengths.

- Similarity

- Betweenness

  - A highly central node (SimBet routing) may use “similarity centrality” and “betweenness centrality” to avoid unnecessary information exchange in the entire network and determine the next hop. A source node with no prior knowledge of the destination node will forward a message to a more central node that has the potential of finding a suitable relay node. A central node has the ease of connecting other nodes in a network.

- Tie Strength

  - SimBetTS protocol extends SimBet protocol to combine “betweeness”, “similarity” and “tie strength” centrality for social routing metric which directs the traffic to more central nodes and thus increases the probability of finding optimal relay for delivering packets the messages.

- Retentiveness refers to predicting  node’s  and region’s remaining available storage

- Receptiveness refers to in-network delays which is important indication that a node or a (geo-temporal) region has volume of traffic greater than it can handle.

- Receptiveness

  - Café and CafREP protocols propose a congestion aware mobile social framework for data forwarding over heterogeneous opportunistic networks and propose “retentiveness” and “receptiveness”. With these two new metrics, Café and CafRep successfully predict and decongest hotspots in social networks.

4 DTN Routing protocols (33 marks)

a) With the help of a diagram describe Spray and Wait protocol.

(5)

Marking:

1 point for a general description of Spray and Wait

2 points for a detailed description and a diagram of Vanilla Spray and Wait.

2 points for a detailed description and a diagram of Binary Spray and Wait.

Answer:

Spray and Wait aims to control the amount of replication by placing an upper limit on the amount of times a message may be replicated. This serves to reduce the amount of wasted resources within the network while still increasing the probability of a message delivery which results from multiple message copies.

Two versions of the protocol which have been proposed in order to achieve this are “Vanilla” and “Binary”, and both versions have two phases, the “Spray” phase and the “Wait” phase. The versions differ only in how they disseminate messages during the Spray phase.

- The Spray phase begins when a source node creates a message.

- A fixed number L is associated with the message, which indicates an upper limit on how many copies of a message may exist within the network

- The Vanilla version is the simpler of the two versions of Spray and Wait available.

  - During the Spray phase, a node will forward one copy of the message to L-1 distinct nodes.

  - After each transmission, the node will decrement the L number of copies of its local copy by 1.

  - Once the number of copies reaches 1, it enters the Wait phase

- The Binary version of Spray and Wait differs from Vanilla in the number of copies a node may disseminate.

  - During the Spray phase, a nodes transfers L/2 of its message copies to the receiving node, leaving itself with L/2 copies.

  - Again, once a node has one copy remaining, it will enter the Wait phase.

  - The advantage to this behaviour is that messages are disseminated away from the source node at a much quicker pace

- As mentioned, the Wait phase occurs when the number of copies L that a node may disseminate reaches 1.

- At this point, the node will only forward the message via Direct Transmission, and will carry it until it encounters the destination.

b) What problem with congestion is not solved by Café? Provide an example of a DTN protocol that solves it.

(5)

Marking:

2 points for correctly identifying that sending rate adjustment is not addressed in Café.

3 points for providing a correct example of DTN protocol that solves it and explaining how (1 point for naming CafRep, 2 points for a detailed explanation)

Answer:

Cafe enables intelligent congestion aware adaptive forwarding in DTNs but does not allow changing of the sending rate.

CafeRep (Congestion Aware Forwarding and Replication) is a replication-based protocol which builds upon Café by coalescing the proposed adaptive forwarding algorithm with an adaptive replication management technique.

Similarly to Café, CafRep uses social driven, resource driven and regional driven heuristics in order to apply adaptive forwarding, disseminating messages away from congestion hotspots to quieter parts of the network.

It uses these metrics in order to calculate the nodes own utility, and upon contact compares it with the encountered node’s utility.

Based upon the comparison, the algorithm decides whether or not to forward a message, and likewise how many copies.

c) Density aware Spray and Wait is an improvement of Spray and Wait. What is the key challenge of Density Aware Spray and Wait protocol? Discuss two reasons it can fail.

(6)

Marking:

2 points for briefly describing DAWS and identifying the key challenge.

4 points for discussing the two reasons it can fail (2 points per reason).

Answer:

- DASW (Density Aware Spray and Wait) is a variant of Spray and Wait that utilises adaptive replication in order to select the number of replications a message is allowed, as opposed to the original fixed number.

- DASW calculates dynamic replication number using an abacus that is precomputed from preceding experiments such (done for RollerNet experiment).

- The abacus is built by simulating a number of Spray and Wait algorithms with varying fixed numbers of replications for different average number of nodes.

- For each simulation, the average delay for a message to be delivered is measured.

- This delay is utilised in the abacus by measuring the number of encounters over a 30 seconds period and comparing it to the recorded delays in order to determine the optimal number of copies.

It is not normally possible predict density in real-time a dynamically changing environments and thus to determine the optimal numbers of replica suitable for the particular dynamic scenario.

- While effective, precomputing the abacus in this way means that it is very case-dependant, and may not be useful as a general purpose approach.

- Likewise, only the 30 second window was used in the proposal, and it did not consider how the performance may be affected using other measurement periods.

d) Describe two differences between geographical routing and other DTN routing protocols. Give examples of two application domains where you would use two different types of routing protocols and justify your answers.

(6)

Marking:

3 points for listing and describing differences between geographical routing and other DTN protocols.

3 points for providing and discussing examples of two application domains.

Answer:

Geographical routing:

It uses maps and locations of vehicles in a centralised service that assumes global knowledge which may suffer from insufficient accuracy.

Centralised services inform the vehicles individually and there is no vehicles-to-vehicle communication.

This is traditional routing in the current transport systems and networks.

GeOpps (Geographical Opportunistic Routing for Vehicular Networks) is:

- a single-copy based distributed protocol

- utilises data gathered from navigation systems in order to route packets towards a specific geographical location.

The next hop of a message is selected as the vehicle that is calculated to be able to deliver the message either to a location closest to the destination, or to the destination in the shortest time

- Each neighbouring vehicle that is potential next hop to the destination, calculates the nearest point they will be able to deliver the packet.

- Each node uses local map and a utility function, to calculate the minimum estimated time the message would take to reach its destination from the nearest deliverable point.

- This is the future protocol which can be used in urban and rural areas and will be more accurate and adaptive to any real time conditions.

e) Describe open loop decision making process in DTNs. How can it improve the problem of congestion?

(5)

Marking:

2 points for describing open loop decision making process in DTNs.

3 points explaining how it can improve the problem of congestion and discussing the mechanisms it utilises.

Answer:

Open loop assumes decision making process in the absence of end to end feedback. This can be very effective with in-network storage and in-network delay monitoring and predictive analytics such as CafeRep.

Retentiveness refers to predicting a node’s region’s remaining available in-network storage.

Receptiveness refers to a node’s in-network delays which is important indication that a node or a (geo-temporal) region has volume of traffic greater than it can handle.

Interest/Content Likelihood, Congesting and Disconnection Rate signals predict how likely it is that a node/region is likely to be interested, congested, disconnected.

f) Explain different motivations for the use of DTNs in urban and rural scenarios.

(6)

Marking:

3 points for listing DTN usage motivations in an urban scenario.

3 points for listing DTN usage motivations in a rural scenario.

Answer:

Urban

- Bulk data distribution in urban areas

- Sharing of individual content in urban areas

- Mobile location-aware sensing applications

- Social Mobile Applications

Rural

- Providing Internet connectivity to rural/developing areas,  e.g. e-mail

- Characteristics

- Intermittent Connectivity

- Mobility, sparse deployment

- High Propagation Delay

- Asymmetric Data Rate: heterogeneous

5. DNS and Information Centric Networks (33 points)

a) A domain in DNS is served by multiple nameservers. Describe the two main benefits from having multiple nameservers. What factors determine whether these multiple nameservers will in practice deliver these benefits?

(4)

Marking:

2 points for describing the two main benefits.

2 points for describing the factors determining the usefulness of the benefits.

Answer:

Scalability and redundancy in the face of server and network failures.

The multiple servers should fail independently and therefore should be located in different geographic locations and have separate connections to the Internet.

b) A local DNS server typically discards cached name-to-address mappings when the time-to-live expires.  Alternatively, the local DNS server could optimistically issue a new query for the cached domain name.  Give one advantage and one disadvantage of the latter approach.

(3)

Marking:

2 points for listing one advantage and one disadvantage (1 point per item).

1 point for providing a further discussion / comparison for the above.

Answer:

By “prefetching” the name-to-address mapping, the local DNS server can hide the DNS look-up delay, improving the performance experienced by the end user.  However, prefetching introduces extra DNS queries and network load to look up name-to-address mappings that may never be needed.

c) Is there a way to perform a denial of service attack with the help of DNS servers?  Please justify your answer.

(3)

Marking:

1 point for answering correctly that there is a way to perform a DoS using DNS servers.

2 points for justifying the answer and providing a brief explanation.

Answer:

Yes.

An attacker can perform a DOS by sending multiple simultaneous queries to multiple DNS servers with a spoofed source IP address (that of the target). This is possible because DNS usually uses UDP. UDP diagrams’ source IP address is easier to spoof compared to that of TCP ones.

d) Explain two important challenges that Information Centric Networks address over DTNs.

(4)

Marking:

4 points for listing two challenges and briefly explaining how they are addressed: 2 points for listing two challenges - 1 point per challenge, 2 points for briefly explaining the two challenges are being addressed - 1 point per challenge addressed.

Answer:

- Accessing named resources – not hosts

  - Named Data Objects (NDOs) rather than IPs  or URLs

  - This allows high scalability and good control of resolution/routing and access.

- Decoupling of  senders from receivers

  - Using asynchronous distributed P2P rather than end to end communications

  - With in-network caching - for all applications, users and service providers (not only for some)

  - Multi-party communication through replication

  - This allows higher reliability and scalability in mobile topologies.

e) Describe two different address structures in Information Centric Networks (ICNs).

(9)

Marking:

2 points for listing two different address structure in ICNs (1 point each).

6 points for describing the two different address structures in detail (3 points each)

1 point for mentioning any of the main characteristics of ICNs.

Answer:

Data address structures:

- Content Centric Networks

  - Naming: Hierarchical naming, single address

  - Security: Signed content

  - Routing: Longest prefix matching

  - Caching: Local or network based

  - Content existence knowledge: Not part of the CCN core

  - Producer-consumer meeting: Propagation of interests

- Publish Subscribe Internet Routing Paradigm

  - Naming: Multi-level identifiers

  - Security: Signed content

  - Routing: (1) Name resolution (2) Information transfer

  - Caching: Network based

  - Content existence knowledge: Registrations in Rendezvous system

  - Producer-consumer meeting: Rendezvous system provides location

Main characteristics of ICNs:

- Change of network abstraction from “named host” to “named content”

- Security built-in: secures content and not the hosts

- Mobility is present by design

- Can handle static as well as dynamic content

f) Explain Information Centric Networks (ICNs) forwarding, transport and caching.

(6)

Marking:

6 points for explaining forwarding, transport and caching (2 points each).

Answer:

Basic ICN forwarding:

- Consumer ‘broadcasts’ an ‘interest’ over any & all available communications media

- Interest identifies a collection of data - all data items whose name has the interest as a prefix.

- Anything that hears the interest and has an element of the collection can respond with that data

Basic ICN transport:

- Data that matches an interest ‘consumes’ it.

- Interest must be re-expressed to get new data. (Controlling the re-expression allows for traffic management and environmental adaptation.)

- Multiple (distinct) interests in same collection may be expressed (similar to TCP window).

Caching:

- Storage and caching are integral part of the ICN service.

- All nodes potentially have caches; requests for data can be satisfied by any node holding a copy in the cache.

- ICN combines caching at the network edge as in P2P and other overlay networks with in-network caching (e.g., transparent web caches)

g) Describe one core challenge of mobile publishers and one core challenge of mobile subscribers in Information Centric Mobile Delay Tolerant Networks.

(4)

Marking:

4 points for describing the two individual challenges (2 points each).

Answer:

Mobile publishers can cache chunks of the content the produce in different distributed caching nodes. The aim of the publishers is to always provide data as close as possible to the subscribers in order to minimize delays and network traffic. “Where” to cache is a very challenging problem when the publishers are mobile and do not how their subscribers are. State of the art research combines social forwarding and DTNs in order to forward the chunks via shortest paths to the intermediary nodes which are the closest to the subscribers.

Mobile subscribers need to register their interests in the remote content either in the intermediary nodes or in the content publishers. As subscribers move, the main difficulty with the registering interests with (possibly mobile) intermediate nodes is that these nodes may be very distant from the subscribers. “How far away to register interest” is a very challenging problem when the subscribers are mobile. State of the art research combines ICNs and DTNs designs to direct interest advertisements very well and to avoid flooding.

END OF EXAM